RULES AND REGULATIONS OF

SOUTHWEST SUBURBAN DENVER

WATER AND SANITATION DISTRICT

2015 Issue

SECTION 1:	PURPOSE AND POLICIES	1-1

SECTION 2:	DEFINITIONS	2-1

FOOTNOTE KEY	2-4

SECTION 3:	AVAILABILITY OF SERVICE	3-1

SECTION 4:	VIOLATIONS, DISCONTINUATION OF SERVICE,

ENFORCEMENT AND COLLECTIONS	4-1

ARTICLE I	VIOLATIONS	4-1

ARTICLE II	DISCONTINUATION OF SERVICE	4-1

ARTICLE III	ENFORCEMENT AND COLLECTIONS	4-2

SECTION 5:	MAIN EXTENSION PROCEDURES AND GENERAL

REQUIREMENTS FOR PLAN REVIEW, CONSTRUCTION

AND ACCEPTANCE	5-1

ARTICLE I	PLAN SUBMITTAL AND REVIEW	5-1

ARTICLE II	EASEMENT REQUIREMENTS	5-2

ARTICLE III	SERVICE LINE STUB-INS	5-4

ARTICLE IV	CONSTRUCTION PROCEDURES AND GENERAL REQUIREMENTS

5-5

ARTICLE V	REIMBURSABLE COSTS	5-6

ARTICLE VI	ACCEPTANCE PROCEDURES AND GENERAL REQUIREMENTS

5-7

SECTION 6:	SANITARY SEWER SERVICE	6-1

ARTICLE I	PROCEDURES FOR APPLICATION FOR PURCHASE OF SANITARY SEWER TAP PERMITS	6-1

ARTICLE II	CONSTRUCTION PROCEDURES AND GENERAL REQUIREMENTS

6-2

SECTION 6:	SANITARY SEWER SERVICE (continued)

ARTICLE III	USE OF THE SEWER SYSTEM	6-4

ARTICLE IV	UNAUTHORIZED TAPS	6-4

ARTICLE V	SINGLE-FAMILY ATTACHED, CONDOMINIUMS AND

TOWNHOUSES	6-5

ARTICLE VI	APARTMENTS, COMMERCIAL AND INDUSTRIAL PROPERTIES

6-5

ARTICLE VII	GREASE AND OIL REMOVAL SYSTEMS	6-5

ARTICLE VIII	ANNUAL SEWER SERVICE FEES	6-8

ARTICLE IX	DRY CLEANING FACILITIES	6-9

ARTICLE X	MOLYBDENUM DISCHARGES	6-9

SECTION 7:  WATER SERVICE	7-1

ARTICLE I	LICENSES FOR WATER SERVICE	7-1

ARTICLE II	FEES AND CHARGES	7-1

ARTICLE III	APPLICATION PROCEDURES FOR OBTAINING A WATER

SERVICE LICENSE	7-2

ARTICLE IV	SUSPENSION, REVOCATION AND CANCELLATION OF LICENSES

7-3

ARTICLE V	WATER SERVICE CHARGES	7-3

ARTICLE VI	SERVICE LINES AND TAPS	7-4

SECTION 8:  EXHIBITS	8-1

A.	APPLICATION AND AGREEMENT FOR EXTENSION OF WATER MAINS

B.	APPLICATION AND AGREEMENT FOR EXTENSION OF SEWER MAINS

C.	SANITARY SEWER EASEMENT DEED

D.	APPLICATION FOR SEWER TAP PERMIT

E.	APPLICATION FOR SANITARY SEWER STUB-IN PERMIT and EXHIBIT A

F.	DENVER WATER DEPARTMENT APPLICATION FOR WATER SUPPLY LICENSE OUTSIDE THE CITY AND COUNTY OF DENVER

G.	LICENSE AGREEMENT FOR CROSSING EXISTING SEWER MAINS

H.	UTILITY TRANSFER FORM

I.	BILLING INFORMATION CHANGE REQUEST FORM

SECTION 9:  SPECIFICATIONS AND STANDARD DETAILS 	9-1

SECTION 1

PURPOSE AND POLICIES

These Rules and Regulations have been enacted to serve the public in securing the health, safety, and general welfare of the inhabitants of the Southwest Suburban Denver Water and Sanitation District and to assist the District’s Board of Directors in the administration and operation of the District.  The sole authority for the operation of the District is vested in the Board of Directors by the Colorado Special District Act, C.R.S. Sections '32-1-101 to '32-1-1307 and '32-4-113.

The District is subject to the Rules and Regulations of the Denver Water Board, Metro Wastewater Reclamation District and other governmental regulatory agencies.  Compliance with the District’s Rules and Regulations does not ensure compliance with other regulatory agencies’ requirements.

Any ambiguity, conflict, omission or question of interpretation will be determined at the discretion of the Board.  Further, the Board reserves the right to change, alter or amend these Rules and Regulations as it deems necessary.  Any such alterations, additions or amendments shall be binding and of full force and effect as of the date such changes were approved by the Board of Directors.  An asterisk,  " * ", by a section number or a subsection denotes a revision or addition has been made effective the date indicated on the Footnote Key page.  An asterisk, " * ", by an Article title indicates the entire Article has been added or changed effective the date indicated on the Footnote Key page.

SECTION 2

DEFINITIONS

As used in these Rules and Regulations, unless the context shall otherwise require, the words defined in this section shall have the meanings herein ascribed:

- ACTUAL COST:  All direct costs applicable to the construction of a given main, including construction, engineering, legal, inspection, plan approval fees and other costs.

*6

- AVERAGE ANNUAL WATER USAGE (AAWU):  Annual average water usage in gallons is  determined by adding together the three lowest monthly water meter readings taken from a location by Denver Water for the 12 months preceding  the District’s annual budget hearing and multiplying the sum by four.

- BIOCHEMICAL OXYGEN DEMAND (BOD):  The quantity of oxygen utilized in the biochemical oxidation of organic matter under standard laboratory procedure in five (5) days at twenty degrees Centigrade, expressed in milligrams per liter.

- BOARD:  The elected or appointed Board of Directors of the Southwest Suburban Denver Water and Sanitation District.

- CROSS CONNECTION:  A physical linkage permanently or temporarily connecting a water source of a lesser quality than prescribed in the Colorado Department of Health’s Drinking Water Regulations to the distribution system serving a public water system.

- DENVER: The terms ADenver@ and ACity of Denver@ whenever used in these Rules and Regulations are synonymous and refer to the City and County of Denver.

- DENVER WATER: The terms "Denver Water", "Water Department", and "Denver Water Department" whenever used in these Rules and Regulations are synonymous and refer to that agency under the control of the Denver Board of Water Commissioners as defined by Section C4.19 of the Charter of the City and County of Denver, upon which this District relies for its supply of water and maintenance and operation of the water system within the District boundaries.

- DENVER WASTEWATER: The terms ADenver Wastewater,@ AWastewater,@ and “Denver Wastewater Management@ whenever used in these Rules and Regulations refer to Denver Wastewater Management.

- DISTRICT:  The Southwest Suburban Denver Water and Sanitation District.

- ENGINEER:  Person or firm that has contracted to do engineering work for the District including but not limited to engineering design, field inspections, surveying and other engineering related services.

- FEE SCHEDULE:  A list of current fees charged by the District.  Fees are subject to change without notice.  The Fee Schedule can be obtained by contacting the District’s Management Company.

- GARBAGE:  Solid wastes from the domestic and commercial preparation, cooking, and dispensing of food, and from the handling, storage, and sale of produce.

- LICENSED PLUMBER OR UTILITY CONTRACTOR:  A person who has been approved to work in the District and who is licensed by applicable municipality or the State of Colorado.

- MANAGER:  The person duly appointed by the Board to administer the affairs of the Board as well as the Board's employees, if any.

- METRO WASTEWATER RECLAMATION DISTRICT (Metro Wastewater):  The term "Metro Wastewater" whenever used in these Rules and Regulations shall mean the Metro Wastewater Reclamation District which is responsible to the District for continuation of transportation of sanitary waste and the treatment thereof.

- NATURAL OUTLET:  Any outlet into a watercourse, pond, ditch, lake, or other body of surface or ground water.

- NON-RESIDENTIAL WASTES:  The liquid wastes from non-residential processes, trade, or business as distinct from domestic sanitary sewage.

- OWNER-DEVELOPER:  The builder(s), landowner(s), developer(s), subdivider(s), agency(ies), or person(s) actually paying for the construction of mains, and/or  physical tap permits.

- PERSON: Shall mean any individual, firm, corporation, association, company,  group or society.

- pH:  The common logarithm of the reciprocal of the mass of hydrogen ions in grams per liter of solution.

- SANITARY SEWER SYSTEM:  All facilities for collecting, storing, transporting, pumping, treating, measuring, etc., sewage.

- SERVICE LINE:  Any pipe, not owned by the District, used to provide water or sewer service to a building or premises from the District's water or sewer mains.

- SEWER MAINS:  Any pipe, piping or system of piping 8 inches in diameter or larger used as a conduit for the sewage in the District's sanitary sewer system.

- SEWER MAIN EXTENSION:  Any pipe, piping or system of piping 8 inches in diameter or larger used as a conduit for the sewage in the District’s sanitary sewer system in addition to the existing sewer system.

- STUB-IN: A service line connected to a District sewer or water main, for which a tap permit and use of the system has not been granted, and which extends to any point from the property line to within five feet of the building foundation for single family residential use, and for commercial and multi-family use, which extends to any point from the property line to inside the building foundation, to which no inside building plumbing fixtures have been connected.

*6

- SYSTEM DEVELOPMENT FEES:  Charges assessed to an applicant by both the District and Metro Wastewater Reclamation District for permission to connect to and use the District's sanitary sewer system which is connected to the Metro Wastewater Reclamation District’s treatment plant through contracts with the District.  Charges are assessed upon application for a new tap or change of an existing tap and are based on the size of the water meter for the facility. These charges may be used to finance system improvements and additions, including but not limited to capital improvements and replacements, or for capital reserves, or for the retirement of debt incurred for existing facilities.  The District collects fees on behalf of both entities and distributes the funds accordingly.

- TAP OR CONNECTION:  The physical connection of a service line to a main.

- USER:  Any person or entity to whom water and/or sewer service is provided, including renter, record owner, corporation, company individual, etc.

- WATER MAINS:  Any pipe, piping or system of piping 8 inches in diameter or larger, used for distributing water along public streets or appropriate rights-of-way or easements directly to various licensed premises within the District's boundaries.

- WATER SYSTEM:  The plant, facilities, system and assets controlled by the Denver Water Department.

SECTION 2

FOOTNOTE KEY

*1	October, 1990 Revisions - Name change only of Metro Sewerage Disposal District #1 to Metro Wastewater Reclamation District #1.

*2	January, 2008 Revisions – Complete reorganization and revision of the entire Rules & Regulations.

*3	July, 2008 Revisions

*4	January, 2009 Revisions

*5	July, 2009 Revisions

*6	January, 2011 Revisions

*7	January, 2012 Revisions

*8	January, 2013 Revisions

*9	January, 2014 Revisions

SECTION 3

AVAILABILITY OF SERVICE

The District has limited capacity for single-family or equivalent water and sewer service taps within its boundaries.

The District will certify to availability of water and sewer service to the extent of its capacity for single-family dwelling units.  Certification of availability of water and sewer service for all multi-family dwelling units, commercial and industrial users will be reviewed by the Board of Directors upon individual application therefor.

Inclusion of land into the District boundaries must be approved by the Board of Directors, and the District may warrant availability of water and sewer service to lands so included.

The District does not warrant the delivery of specific quantities of water, nor does it undertake to maintain water pressure sufficiently high enough to operate sprinkling systems, automatic household appliances, or other equipment dependent on water pressure for their operation.  No allowance will be made by way of reduction on demand, contract, readiness to serve, or minimum charges for reduction in quantities of water normally delivered caused by failure of supply, line breaks, or restrictions placed on use of water during times of shortage or low pressure.  During periods of short supply, the District will act as an agent for all of its users and endeavor to ensure each a fair and equal share of the water available.  To this end, the District may impose sprinkling restrictions or other water conservation measures.

SECTION 4

VIOLATIONS, DISCONTINUATION OF SERVICE,

AND ENFORCEMENT & COLLECTIONS

ARTICLE I

VIOLATIONS

*6

- Any person found to be violating any of the provisions of these Rules and Regulations shall be served with written notice stating the nature of the violation and generally providing a reasonable time limit for satisfactory correction thereof.  In an emergency situation a verbal notice may be given to the violator followed by a written notice.  All notices that are required by statute to be mailed certified, return receipt requested shall be mailed in such manner.

Any person violating any of the provisions of the Rules and Regulations shall become liable to the District for any expense, loss or damage occasioned by reason of such violation and for any fines imposed.

Violations of any of the provisions of the District’s Rules and Regulations may also result in the discontinuation of water service depending on the nature of the violation.   Each violation will be reviewed by the District’s Board and a determination made as to what action will be taken by the District.

ARTICLE II

DISCONTINUATION OF SERVICE

- Upon compliance with the procedures set forth herein, the District may discontinue water and/or sewer service to any parcel of real property for nonpayment of system development fees, service fees or similar charges, or for failure to comply with any of the requirements of the District’s Rules and Regulations.

- The following procedure will be adhered to prior to the discontinuation of water service for nonpayment of charges:

*7

- When any charges have become delinquent,  the District will mail or deliver to any person (or entity) liable therefore, at the service address and known current mailing address, a notice (the "Notice") advising such person (i) of the amount claimed to be due, including any delinquent charges; (ii) that water service will be discontinued for nonpayment of charges 5 days after notice is received, unless full payment of all amounts due is received before that date; (iii) that said person has the right to a hearing concerning the alleged delinquency or the correctness of the amount claimed prior to discontinuation; and (iv) that said person must request such hearing at least 2 days before the discontinuation date specified in the Notice.

*7

- If the responsible party fails to request such hearing within the time provided, the District will arrange to have the water service for the premises for which charges are delinquent discontinued forthwith and assess a water turn off fee that must be paid in addition to all other past due amounts.

*7

- If the responsible party makes timely request, the District’s Manager will promptly notify the responsible party of the date of the next regularly scheduled board meeting at which a hearing will be scheduled.  The responsible party may be represented by counsel at the hearing.  The scope of inquiry at such hearing will be limited to whether the charges are delinquent and whether the amount claimed is in fact owed, in full or in part.  The Board will state their decision in writing.  Discontinuation of water service will be stayed until the Board holds the hearing and renders the decision.

*7

- All amounts found to be due and owing by the District’s Board will be paid on the same business day that the Board renders its decision; provided, however, that upon an adequate showing of mitigating circumstances by the responsible party, the Board may extend the stay for up to ten (10) days following the date of the decision.  If payment is not made as required herein, the District will arrange to have the water service for the premises for which charges are delinquent discontinued forthwith and assess a water turn off fee that must be paid in addition to all other past due amounts .

*7

- All past due accounts must be paid in full or the amounts found to be due by the Board must be paid in full.  The District will then notify the Denver Water Department to allow resumption of service.  In addition to paying the District’s water turn on fee, the owner may be required to pay a reconnection fee to Denver Water to have service reconnected.  Reconnection will be made in accordance with the District’s agreement with the Denver Water Department.

ARTICLE III

ENFORCEMENT AND COLLECTIONS

- All fees and charges imposed by the District will become a lien as set forth in section 4.302A below, on the property to which services are provided from the date said charges become due until said charges are paid.  The property owner will be liable for all fees and charges for uses on his or her premises.  In case the tenant in possession of any premises shall pay the charges due, it shall relieve the owner from such obligations and lien, but the District hereby assumes no responsibility for any agreements between landlords and tenants, regardless of how made or whether the District was notified of such agreements and shall not be required to seek payment from any person whomsoever other than the owner.  No change in ownership or occupation shall affect the application of this Section and the failure of any owner to discover that he or she purchased property against which a lien exists shall in no way affect his or her liability for such payment in full.

- In addition to any other remedies available to the District, including the remedy of discontinuation of water service set forth in Article II above, the District may take the following action upon failure to pay any charges or fees by the date specified as due upon the bill or when they are otherwise due:

  - The District may foreclose the perpetual lien placed on and against the property served in the same manner as provided by the law of the State of Colorado for the foreclosure of mechanics' liens (C.R.S. '38-22-101, et. seq.).  The District shall take all necessary and proper steps to assure the enforcement and payment of charges assessed pursuant to this Article.

  - The District may maintain an action for the amount of charges due in a court of competent jurisdiction including interest as allowed by law.

  - The District may certify the amount of any charge due to the County Treasurer to become an assessment upon the property served to be collected in the same manner provided by law for the collection of general property taxes.

SECTION 5

MAIN EXTENSION PROCEDURES AND GENERAL REQUIREMENTS

FOR PLAN REVIEW, CONSTRUCTION AND ACCEPTANCE

ARTICLE I

PLAN SUBMITTAL AND REVIEW

- It shall be a violation of District policy for any person to construct a water and/or sanitary sewer main within the jurisdiction of the Board without first having written approval of the Board of Directors of the District.

- All plans shall be in compliance with the Engineering Standards and Specifications of the Southwest Suburban Denver Water and Sanitation District.

- The owner/developer is responsible for payment to the District of all fees and charges associated with the plan review process, including the charges of the District Engineer, District Manager and District Attorney.

- No construction may take place until the plans have been reviewed and preconstruction meeting has been held.

- Plans and specifications are approved for a six-month period only.  If construction has not begun within this six-month period, or if it has been halted and not restarted prior to expiration of the approval period, the plans must be resubmitted for review and approval

- The owner, developer or design engineer may request a pre-design conference by contacting the District Manager.  This conference is not required but will be held upon the request of the owner, developer or design engineer.

- Preliminary sewer plans shall be submitted in three copies to the District Engineer.  The plans will be reviewed by the District Engineer for compliance with the District's Standards and Specifications, as well as for compliance with the District's integrated sewer system.  The approval of plans will also be subject to the ability of the District to operate and maintain the proposed facilities in an efficient, economical manner.  Plans to be reviewed must be accompanied by three (3) sets of recorded plats.  One copy shall be retained by the District Manager, one copy delivered to the District Engineer and one copy delivered to the District’s Attorney.

- Two (2) original sets of the District's Application and Agreement for Extension of Sewer Mains shall be submitted to the District Manager prior to plan review approval

- If any sewer main is to be constructed outside of a public right-of-way, the construction plans must be accompanied by a request for acceptance for a sewer easement.  All easement information, in accordance with these specifications, shall be submitted to the District Engineer prior to plan review.

- Upon completion of the District's review of preliminary plans, one copy of the plans will be returned to the design engineer for required revisions.  A conference may be scheduled by the District Engineer to discuss the needed revisions if it is deemed necessary or if it is requested by the design engineer.

- When all revisions have been completed, three copies of the revised plans shall be submitted to the District Engineer who will coordinate the review with the District Manager and other review agencies.

- If no further revisions are required by the District, four (4) copies of the plans shall be submitted to the District Engineer for approval.

- Upon final acceptance of construction plans, the approved drawings shall be held by the District Engineer and released at the preconstruction meeting.  NO construction may take place until a preconstruction meeting has been held.

- All water and sewer main extension shall be constructed according to the District's specifications, and those of Denver.  WARNING:  In many instances, the District imposes stricter requirements than those established by Denver Water Board or Metro Wastewater Reclamation District.  In any instances of conflict the District's requirements shall be followed.  Any questions should be referred to the District’s Engineer.

- Locations of Water and Sewer Main Extensions and Additions:  Water and sewer mains shall be installed in roads or streets which the County, State Highway Department or other public agency has accepted for maintenance as a public right-of-way, as well as in easements granted to the District.

ARTICLE II

EASEMENT REQUIREMENTS

- Each landowner, subdivider or developer who desires water and/or sewer service will, in consultation with and approval of the District, plat and grant to the District appropriate easements and right-of-way.  The District may require appropriate easements and rights-of-way for its own use, or where facilities to be constructed are required to cross land not being subdivided or under the landowner, subdivider or developer's control.

- Easements are required wherever a sewer main is not in a public right-of-way.  All easements shall be prepared according to the following specifications.

In areas where sewer mains are installed in easements, the sewer mains shall be located within the easements as shown on the construction plans.  All sewer main easements must be a minimum of thirty feet (30') in width.  No sewer main shall be located less than five feet (5') from the edge of an easement.

All easements granted to the Southwest Suburban Denver Water and Sanitation District shall be prepared by the easement grantor or his designated representative according to procedures set forth.

- Procedures

  - These procedures have been developed to unify the submittal of information required for the preparation of sewer easements.  The following information shall be submitted to the District Engineer in four copies:

    - A separate legal description of each easement is required for each separate ownership.

    - A drawing of each easement on an 8-1/2" x 14" sheet showing distances, north arrow and ties to recognize land corners.  The drawing shall be prepared on a scale of one inch equals fifty feet.  A separate drawing is required for each legal description.

    - A general location map of the development area and easements on an 8-1/2" x 14" sheet.

    - A title commitment or title policy on the entire tract of land to be developed.

    - A boundary survey of the tract of land to be developed showing the location of the proposed easement(s), as well as the location of all structures, ditches, existing easements and other encumbrances explained in the title commitment

    - A title commitment for each separate ownership on land traversed by all offsite easements specific to the easement legal description.

- Construction Within Easements

  - Plans for sewer main construction within easements shall not be reviewed nor shall construction be authorized prior to the acceptance of the easement by Southwest Suburban Denver Water and Sanitation District.

  - Prior to the acceptance of sewer mains within easements, the District shall be provided with a drawing indicating the “as constructed" location of the sewer main within the recorded easement.  This drawing must be certified by a registered land surveyor.

- License to Cross Southwest Suburban Denver Water and Sanitation District Easement

  - In the event that it is necessary for another utility to cross the existing easement dedicated to the Southwest Suburban Denver Water and Sanitation District, it will be necessary to obtain a license agreement (See Exhibit G).  The procedure for crossing an existing easement is presented below.

Prepare a legal description of the precise location of the crossing, consisting of a map and printed legal description.  The legal description should tie to a land corner or other recognizable point in a platted subdivision.

- In the event the landowner, subdivider or developer employs the engineering services of an engineer or engineering firm that is retained by this District as its District Engineer, then the Board reserves the right to have the engineering services, design, etc., reviewed by an independent engineer.  Expense of such review shall be borne by the landowner, subdivider or developer.

ARTICLE III

SERVICE LINE STUB-INS

- Service line stub-ins may be permitted from the main past the curb for sewer, provided the owner-developer or contractor either applies for the stub-ins with the Application and Agreement for Extension of Mains or makes a separate application for stub-ins prior to the time of building and connecting to an existing sewer main.

- If service line stub-ins are connected to a sewer main at the time the main is constructed, additional inspection fees will not be charged.  The owner-developer will be charged fees as required by the Application and Agreement for Extension of Mains and a permit fee for each stub-in.  If a request is made for a service line stub-in to an existing sewer main, the owner-developer must complete the District’s Agreement for Sanitary Sewer Stub-In Permit Application (See Exhibit E) and pay the District's current stub-in permit and inspection fee.

- The District will allow stub-ins to be extended from the main as follows, depending on the type of use:

A.	Single Family Residential Units (Detached/Attached/Patio Homes) - Stub-ins will be allowed to extend to any point inside the property line, within five feet of the outside wall of the building foundation.  Service lines may not be extended from the stub-in to inside the building foundation unless an Application for Sewer Tap Permit has been submitted along with payment of all applicable fees.

B.	Multi-Family Housing (Apartments/Condos/Townhouses) - Stub-ins will be allowed to extend to any point inside the property line, including inside the building foundation.  Service lines may not be extended from the stub-in to inside the building foundation, nor may inside plumbing and fixtures be connected to the stub-in, unless an Application for Sewer Tap Permit has been submitted along with payment of all applicable fees.

  - Commercial Buildings (Single Tenant and Multi-Tenant Buildings) - Stub-ins will be allowed to extend to any point inside the property line, including inside the building foundation.  Service lines may not be extended from the stub-in to inside the building foundation and/or connection of inside plumbing and fixtures may not be made unless an Application for Sewer Tap Permit has been submitted along with payment of all applicable fees.

Upon completion of the stub-in permit application and payment of the fees by the owner-developer, the District Manager will approve and issue the permit for stub-in purposes only.

- Upon request for tap connection of the building service line to the stub-in or connection of building plumbing and fixtures to the stub-in, the owner-developer or contractor must make an application for a sewer tap permit and pay the District's then existing system development and inspection fees and all fees payable to the other entities as provided in Article 3 of this Section.

- Use of a stub-in service line or connection of building plumbing and fixtures to a stub-in is prohibited.  The District may impose fines of up to $500 per day from the time the violation is discovered until such time as the stub-in is converted to a tap by submitting the necessary tap application and fees.

##### ARTICLE IV

CONSTRUCTION PROCEDURES AND GENERAL REQUIREMENTS

- The contractor shall be responsible for arranging a preconstruction meeting prior to the start of any construction.  The District Engineer, District Manager, contractor and owner or owner's engineer must be represented at this meeting.  Other representatives should include the soils engineer and surveyor.

- The contractor shall be responsible for notifying the District Engineer and District Manager at least 48 hours prior to start of any construction.  If work is suspended for any period of time after initial start up, the contractor must notify the District at least 24 hours prior to restart.

- A Performance and Maintenance Bond equal to 100% of the contract (or construction cost) shall be furnished by the contractor to the District on all sewer main construction in the District.

- Prior to the start of any work where sewer mains are to be installed into existing District sewer system the nearest manhole to the point of tie-in shall be plugged with a plumbers plug on the outlet side by the: contractor.  This plug shall remain in place until acceptance of construction by the District.  Its purpose is to prevent any mud, water or other materials from entering the existing line during construction.  The contractor shall be responsible for pumping and cleaning manholes and removing the plug.

- At the request of the District, no pipe or appurtenance shall be backfilled, nor covered with bedding material, above the spring line of the pipe prior to the field observation by the District.  Arrangements shall be made by the contractor to assure that all construction is reviewed by the District prior to backfilling.  Any pipe covered prior to acceptance shall be excavated by the contractor to allow for review.  This shall be done at no expense to the District.

- Approved plans and a copy of the specifications must be kept on the job site by the contractor at all times.  The contractor shall maintain a marked up set of plans showing any variations of the contract drawings.  The contractor is responsible for maintaining record drawings to include all distances between manholes and locations of wyes and service tees.  Record drawings (including two copies of blue line prints and one electronic copy) must be reviewed by the District before probationary acceptance and lamping of the line. The final record drawings should be a minimum of  12” x 18" and provided in electronic format as required by the District’s Engineer.

- All construction, including labor, materials and settlement of backfill, shall be guaranteed by the owner/ developer for a period of one (1) year from the date of acceptance of construction by the District Manager and District Engineer.  During the guarantee period, the expense of any repairs or maintenance to the lines, appurtenances and facilities shall be the responsibility of the owner/developer.  The District reserves the right to ensure proper operation of its system and to perform any cleaning, repairs or other maintenance during the warranty period at the expense of the owner/developer.

- The date of final acceptance will be as indicated in the conditions of the application for sewer main extensions.

ARTICLE V

REIMBURSABLE COSTS

- The owner/developer will be charged for all reasonable costs incurred by the District in connection with a main extension project for services performed by the District’s Engineer, Attorney, Manager and other agents in accordance with such persons’ then current fee schedules. The services will include but not be limited to the following items of work when applicable:

  - Review of plans and specifications in accordance with the district plan approval procedures provided herein.

  - General and resident supervision and field observation of work of the contractors, all line and grade surveys as construction progresses.  The field observation is not to be continuous, but daily will include: (1) measurement for payment, (2) assistance in interpretation of plans and specifications, (3) accurate daily field records of work performed, difficulties encountered, etc., and (4) protection of the District's interests, but not actual field checking of alignment, joints, etc., except where not performed by the County’s inspectors.

  - All daily inspection fees of water or sewer mains, including but not limited to those required by Metro Wastewater Reclamation District, Jefferson County, the Denver Water Board or the State Highway Department shall be paid for by the plumber contractor or others requesting main extensions in the District.

  - General administration of the project including, but not limited to, all necessary record keeping, correspondence, and reports, and attendance at all necessary meetings pertaining to each construction project.

  - Review of contracts, proposals, easements, right-of-way descriptions, title commitments, and other legal documents involving the owner-developer’s project and the District.

  - Preparation of required rights-of-way descriptions and drawings for the District, performing necessary surveys for these rights-of-way, but not procurement of same from the owners.

  - All necessary meetings with the Board pertaining to the particular contracts.

  - Revision of permanent District maps.

  - Removal of debris and jet cleaning of all sewer mains for conditional and final acceptance.

  - TV’ing and a video tape made of the TV inspection for the District’s records.

- All costs will be assessed monthly, with the final costs being assessed after all construction inspections have been completed and the main(s) have been conditionally accepted by the District Engineer.  The final invoice will include an estimate of the job closeout costs.  Conditional Acceptance of the main(s) by the District is contingent upon the District receiving payment of the final invoice.

- If the final invoice for the project is not paid within 60 days of receipt of the invoice, the entire project will be subject to suspension of sewer and/or water service and the suspension of additional taps being sold for the project until payment of the final invoice is received.

- Any other fees charged to the District by others for the owner-developer’s benefit will be paid for by the owner/developer.

ARTICLE VI

ACCEPTANCE PROCEDURES AND GENERAL REQUIREMENTS

- All water and sewer mains, except service lines, constructed in the District become the property of Denver Water or the District upon final acceptance of the facility.

- The District Engineer performs services on behalf of the District to ensure compliance with the District's Rules and Regulations and Specifications.  Engineering services performed by the District Engineer shall be charged to the developer.  The District Engineer does not represent the landowner, subdivider or developer.  The landowner, subdivider or developer must rely solely on its own engineer to ensure proper design and installation of utilities.  The District Engineer also ensures that the utilities to be installed will meet the District's requirements for overall system operation and integrity.  While the District Engineer is knowledgeable and does provide assistance in meeting the requirements of other governmental entities, such as the Denver Water Department and Metro Wastewater Reclamation District, approval of construction plans by the District Engineer does not ensure approval by any other governmental agency.

- A main shall be accepted by the District and released for taps when the following conditions have been met:

  - The main and all appurtenances have been installed to the satisfaction of the District Engineer, inspector or field engineer and all pertinent as-built notes and measurements have been made and two sets of blue line prints with one electronic verssion of the as-built drawings have been supplied to and approved by the District Engineer.

  - The main has been successfully low-pressure air tested and inspected to the requirements of the District.  Low pressure air testing is mandatory.

  - All compaction test results required by the District have been submitted.

  - All easements have been submitted and recorded.

  - The owner/developer has submitted a letter to the District Manager documenting the installation costs for the project.

  - Drawings have been submitted indicating the "as constructed' location of sewer mains within the boundaries of recorded easements.  These drawings must be certified by a registered land surveyor.

  - The District's applications for sewer main extensions have been dated and signed by the District Manager and District Engineer.

- When the utility lines and as-builts have been accepted, 1 copy of the District’s executed Application and Agreement for Extension of Sewer Mains granting probationary acceptance will be distributed to the Developer

- Prior to the expiration of the probation period, the utility line shall be inspected for final acceptance and maintenance by the District Engineer and District Manager. The developer and his contractor shall correct any deficiencies observed during the final inspection within the time period set by the District personnel.

- Upon final acceptance, the District Manager will distribute a fully executed original Application and Agreement for Extension of Sewer Mains to the Developer.

SECTION 6

SANITARY SEWER SERVICE

ARTICLE I

PROCEDURES FOR APPLICATION FOR PURCHASE

OF SANITARY SEWER TAP PERMITS

- Sewer Service - General:  No connection to a Southwest Suburban Denver Water and Sanitation District owned sewer main, shall be allowed prior to the purchase of a District tap permit and payment of appropriate Metro Wastewater Reclamation District fees.

- Application:  Applications for service must be filed with the District Manager and be accompanied by appropriate fees.  Tap permits will not be issued prior to District preliminary acceptance of the sewer main to be tapped.

- Fees:  The sewer tap or connection fees shall be as shown on the District’s Fee Schedule. All fees may be changed without notice by the Board of Directors.

- All properties requiring water and sewer shall pay the applicable fees charged by the Denver Board of Water Commissioners, Metro Waste Water Reclamation District and Southwest Suburban Denver Water and Sanitation District.

- Cancellation of Application and Refund of Fees:  The District reserves the right to revoke any application previously granted before service has been provided.

- Application for Sewer Service:  All sewer taps must be purchased at the same time as water taps are purchased from Denver Water.

- Denial of Application:  The District reserves the right to deny application for service for any reason, including but not limited to:

  - Connection of the system to applicant's existing plumbing would constitute a cross-connection to and unsafe water supply; or

  - The service applied for would create an excessive seasonal or other demand upon the facilities.

  - Failure to comply with the District's Rules and Regulations.

ARTICLE II

CONSTRUCTION PROCEDURES AND GENERAL REQUIREMENTS

- All cost and expense incidental to the installation and connection of the water or sewer service shall be borne by the landowner, subdivider or developer.  The landowner, subdivider or developer shall indemnify the Board for any loss or damage that may directly or indirectly be occasioned by the installation of the water or sewer service.  No work shall be done by the District on Saturdays, Sundays or holidays unless written permission is granted by the District Manager.

- The inspection fee for sewer main connections shall be as provided on the District’s Fee Schedule.  All permits, fees and licenses shall be paid for by the contractor, plumber or others doing work in the District prior to the start of construction.  No work shall start until the District Engineer’s office has been notified.

- The District Engineer will inspect all sewer taps including stub-ins to the District's sewer mains during construction.  The District Engineer requires a minimum of two working days' notice for inspections.  Inspection should in no way be considered a guarantee of the contractor's work.

- The sewer service line shall be PVC pipe with push on joints or any other material approved by the Board of Directors, subject to the limitations of the Uniform Plumbing Code requirements.  The line shall be watertight and on a constant grade in a straight line, and not closer than five horizontal feet from any bearing wall.

- Sewer service lines shall be installed in a separate trench, maintaining a ten-foot horizontal separation from the water service line.  All sewer service installation shall be in accordance with the specifications of Southwest Suburban Denver Water and Sanitation District, Uniform Plumbing Code, Metro Waste-Water Reclamation District and Jefferson County Building Department.

- No sewer tap connection shall be backfilled until construction has been inspected and accepted by the District Engineer.  The contractor shall be responsible for maintaining safe conditions until such time as all inspections have been completed and the work accepted by the District.

- No connections shall be made to any sewer line of the District, nor shall any service line laterals or extensions be laid or installed except by or under the supervision of a contractor licensed by the District.  Prior to making any connection to the sewer system of the District, installing any sewer lines contemplated to be connected to the District system, or repairing of any sewer service within ten (10) feet of the connection to the District’s sewer main, which are connected to the District main, the contractor must make application for a “Contractor’s License”, which application form will be furnished by the District and approved by the District Engineer.  A licensed contractor must have at least three (3) years of experience in sewer construction.  Before a license can be issued, applicant shall furnish to the District in form and in substance, certificates and copies of insurance coverage that meets the requirements of 6.209. The work on sewer service lines shall be done by approved contractors under the District's supervision, but plumbing work contracted for by an approved plumbing contractor may be performed by  the plumbing contractor through journeyman plumbers or apprentices under their direct supervision and shall meet the Colorado State Department of Health's Technical Plumbing codes.

- Revocation of Contractor’s License: The violation of any of these Rules and Regulations or the District installation specifications shall constitute sufficient grounds for revocation of the contractor's right to continue to work within the District.  Whenever it appears a violation been committed, the contractor shall be sent written notice.

- Liability insurance shall be carried by contractors licensed in the District in sufficient amounts to protect the District against any and all claims that may be occasioned by the work of the plumber or contractor. This insurance shall be written with a limit of liability of not less than $100,000 for all damages arising out of bodily injury, including death, at any time resulting therefrom, sustained by any one person and not less than $300,000 for all damages in any one accident and not less than $500,000 for all damages arising out of the injury to, or destruction of property during the policy period.  Worker's Compensation Insurance shall be carried in accordance with the provisions of the State of Colorado.

- Upon approval of the District Engineer and upon receipt of the above insurance certificates, the  contractor shall pay the license fee in accordance with the District’s current fee schedule and such license shall be effective for the calendar year of issuance, subject to its revocation.  Said license may be renewed for additional periods of one (1) year each January 1st, upon payment of a renewal fee and the furnishing of the required insurance certificates.  Failure to renew the license for three (3) consecutive years shall require a new application to be made.

- All contractors, plumbers and others doing work on any sewer main, service lines or structures in the District shall comply with applicable District, Jefferson County, Denver or State Highway Department regulations on excavation, backfill, compaction and restoration of surfacing.  All excavations required for the installation of sewer service shall be open trench work unless otherwise approved.  Pipe laying and backfill shall be performed in accordance with the standard specifications adopted by the District.

- All construction work and materials shall meet the standards and specifications of Southwest Suburban Denver Water & Sanitation District, Metro Wastewater Reclamation District, the Denver Board of Water Commissioners and the Technical Plumbing Code of the Colorado State Health Department.  The District's specifications may be obtained at the office of the District Engineer. The District's specifications may be more strict than those of other regulatory agencies and shall govern in instances of conflict.

- All construction of sewer facilities shall be adequately guarded with barricades and lights so as to protect the public from hazard.  Street, sidewalks, parkways, and other public or private property disturbed in the course of the work shall be restored in a manner satisfactory to the District.

- Sewer service shall be furnished only to persons whose property is included within the District's boundaries which are subject to the rules, regulations and taxation by the District or as other-wise provided by existing contract.

- Sewer Services - Ownership and Maintenance:

  - Ownership - The sewer service line shall be owned by and installed at the expense of the tap permit purchaser.

  - Maintenance - The customer shall maintain all service pipe and appurtenances, including but not limited to grease traps between the structure being served and the District sewer main.  In those cases where grease traps or other filtering processes are employed, the District reserves the right to periodically review maintenance and schedules in accordance with Section 6, Article VII of the District’s Rules & Regulations.

ARTICLE III

USE OF THE SEWER SYSTEM

- No unauthorized person shall uncover, make any connection with or opening into, use, alter or disturb any District water or sewer main or appurtenances without first obtaining a written permit from the District.

- The owner of the house, apartment or buildings of any nature shall be responsible for repair and upkeep of the sewer or water service lines from the meter or curb stop to the structure in cases of water service, and from the tap on the main to the structure in cases of sewer service.

- Existing water or sewer service lines may be used in connection with new buildings only when found, on examination by the District Manager or District Engineer, to meet all the requirements of these Rules and Regulations.

- A person owning land outside the boundaries of the District who desires service must include all of his land contiguous to the parcel upon which service is desired within the boundaries of the District, unless the Board determines otherwise

*9

- Maintenance of the sewer service lines from the tap on the main to the unit shall be at the property owner's expense and shall be the property owner's responsibility.  Should the service line and/or tap on the main need repair, the owner shall make such repairs within the time frame designated by the District.  Depending on the severity of the repair as determined at the discretion of the District, which may include but not be limited to root intrusion, infiltration from cracks, offset joints or other sources,  the owner may be required to make the repairs within a minimum of four weeks, but in no circumstance shall the repairs not be completed within ninety days of notification by the District.  Shall the repairs not be made within the timeframe designated by the District, the District will proceed to make the repairs and bill the owner for the costs incurred.

- Swimming Pools and Hot Tubs:  No public or private swimming pool or hot tub shall be connected to the sanitary sewer system.  The District does not allow discharge from a swimming pool or a hot tub into its sanitary sewer system.

- The District shall not allow any sub-drain, foundation or groundwater system including sump and sump pumps to be installed within its sewer line trenches and will not allow any such drainage systems to be connected to its sewer lines in any manner whatsoever, either directly of indirectly by discharging into floor drains or other plumbing fixtures or by any other means.

- Where the District has allowed sewer mains six (6) inches or larger to remain the private property of the property owner, such private systems shall be cleaned at least annually by the property owner and more frequently if conditions and use of the mains so dictate.  Manholes on the private systems will be inspected as part of the maintenance program.  Every three (3) years, these lines must be TV’d.  Copies of all cleaning and TV reports must be sent to the District no later than January 31st of each year.

ARTICLE IV

UNAUTHORIZED TAPS

- Any contractor, developer, or other person who makes or causes to be made a tap on the mains of the District without first obtaining written permission from the District will be fined $500.00 per day until an Application for Sewer Tap Permit has been submitted and approved and all fees and fines have been paid.

ARTICLE V

# SINGLE-FAMILY ATTACHED, CONDOMINIUMS AND TOWNHOUSES

- Separate and independent sewer service lines, in conformance with the Jefferson County Building Code or Uniform Plumbing Code, shall be provided for every single-family living unit.  Each condominium, townhouse or single-family family attached structure shall have an individual sewer service line and tap for each living unit in the said structure.

- Except as specifically herein provided, all of the District's Rules and Regulations applicable to single family detached residences shall apply to condominiums, single family attached dwellings, and townhouses constructed in the District.

ARTICLE VI

APARTMENTS, COMMERCIAL AND INDUSTRIAL PROPERTIES

- The number of service lines required to serve sewer for an apartment building will be as established by the District, Jefferson County Building code and Metro Wastewater.

- The District will require a system development fee as listed in the District’s Fee Schedule to be paid for each building.

- All other Rules and Regulations for the installation for service lines in the District will be applicable to the installation of service lines to serve apartment buildings.

- Each non-residential structure hereinafter constructed shall comply with the applicable Rules and Regulations of the District, Jefferson County and Metro Wastewater Reclamation District.  The District’s system development fees will be charged based on the size of the water meter for each building.

ARTICLE VII*3

OIL/GREASE AND OIL/SAND REMOVAL SYSTEMS

- Oil/Grease (O/G) or Oil/ Sand (O/S) removal systems will be provided in all establishments which, as a part of their on-going operations, provide automotive repair and maintenance services, conduct food preparation, food processing, and/or related activities. The O/G and O/S removal systems help remove excess grease, oil and sand before it enters the District’s collection system.   O/G removal systems are typically used for the removal of grease and oil from businesses that prepare food.  They also serve to remove solids which will settle from the waste stream.  O/S removal systems are typically used for the removal of sand and oil from auto maintenance operations.  O/G removal systems are not required for single family residences or multifamily residences, unless the multifamily residences have a common food preparation area.

- By way of illustration, not by limitation, such businesses include restaurants, caterers, fast food establishments, hotels, motels, lodges, "bed and breakfast" facilities, bakeries, grocery stores, multifamily units with common kitchens or similar food preparation and service establishments, car washes, automotive service garages and similar automotive service establishments.

- For the purposes of these Rules and Regulations, "oil  or grease" is defined as animal or vegetable fat, oil, grease, or similar products or by-products having the same characteristics as animal fat, vegetable fat, oil or grease, as well as oil or grease petroleum products.

- General Specifications for O/G and O/S Removal Systems:  All businesses covered by this regulation will be required to install and maintain in good working order, O/G or O/S removal systems upon their respective business premises. All businesses required to install O/G or O/S removal systems shall submit plans and specifications for such systems to the District for its review and approval prior to the installation of any such removal system.  No installed system will be used until the District inspects and approves the installation of such system.  All removal systems will be of a type and capacity approved by the District but, at a minimum, must contain two compartments and must be designed to meet the standards established by the Denver Wastewater Division or the City of Littleton Englewood Wastewater Treatment plant’s Industrial Waste Treatment Division, whichever is more stringent.  All systems will be located so as to be readily and easily accessible for cleaning and inspection and will be installed as follows:

  - All plumbing fixtures and drains contributing waste flows containing oil, grease or sand will be connected to the O/G or O/S removal system, including but not limited to, kitchen and pantry sinks, dishwashers, all food and garbage grinder/disposal units, and all floor drains in areas where food is prepared and utensils are washed or areas where automotive maintenance is performed.  Restroom facilities, showers and water closets will not be connected to the removal system.

  - All O/G and O/S removal systems will be installed in a readily accessible location to facilitate periodic inspection, cleaning and oil, grease and sand removal.  Where possible, the system should be installed in an exterior location, within the property lines of the business or premises, at the upstream end of the private service line.

  - All removal systems will be vented as required by applicable provisions of the Uniform Plumbing Code, which provisions are incorporated herein by this reference.  All removal systems shall have gas tight lids fixed upon each access way to each compartment.

- Maintenance and Cleaning of O/G and O/S Removal Systems.  Individual establishments are responsible for the maintenance and cleaning of their removal systems, as necessary, in order to avoid any obstruction and/or damage to any of the District's sewer facilities.  All  removal systems will be cleaned not less than every three months and more frequently if the following conditions are exceeded:

  - For Oil/Grease removal systems the maximum reduction in capacity of the first compartment shall not be more than 20% and the second compartment shall not have a reduction of more than 5%.  Percent reduction shall be calculated by taking the total liquid depth, the total depth of floating solids and the total depth of settled solids, measured in inches.  The total reduction shall be the sum of the floating solids and the settled solids divided by the total liquid depth of the tank.  The same procedure shall be used for both the first and second compartments.  The total reduction for the entire system shall never exceed 25%. This shall be calculated by adding the reduction of the individual compartments.

  - For Oil/Sand removal systems the maximum reduction in capacity of the first compartment shall not be more than 10% and the second compartment shall not have a reduction of more than 5%.  Oil and floating material shall not be more than 1 inch in the first compartment and no visible floating material in the second department.  Percent reduction shall be calculated by taking the total liquid depth, the total depth of floating solids and the total depth of settled solids, measured in inches.  The total reduction shall be the sum of the floating solids and the settled solids divided by the total liquid depth of the tank. The same procedure shall be used for both the first and second compartments.  The total reduction for the entire system shall never exceed 15%. This shall be calculated by adding the reduction of the individual compartments.

*6

All accumulated material within the removal system including but not limited to oil, grease, sand and other trapped sediments will be completely removed and disposed of in a manner approved by Metro Wastewater and Jefferson County environmental health department.  The business owner or operator shall maintain adequate proof of such cleaning, which proof may consist of services bills or other documentation showing the date and volume of material removed.  Records shall be available for District staff to inspect upon request and shall be available for the last 12 consecutive months.

- Inspection of O/G and O/S Removal Systems.  District representatives shall have the right to make periodic inspections of all removal systems.  An annual inspection fee, as reflected on the District’s Fee Schedule, will be charged to all customers having an O/G or O/S removal system to cover the costs of such inspections.  During an inspection, the District representative may request and the business owner/operator shall provide maintenance records for the removal system.

*6

If upon inspection, any removal system is determined in the discretion of the inspecting District representative, not to be in compliance with either the maintenance requirements or specifications of these Rules and Regulations, the business owner/operator will be given written notification of non-compliance and charged a re-inspection fee.  The District  shall  notify the business owner/operator the time within which all violations will be corrected.

- Violations.  If the violations have not been corrected within the time designated by the District, after written notification of non-compliance has been given, the District may have the business's removal system pumped and/or repaired.  The District shall charge the business owner/operator for the cost of the pumping and/or repair, for any and all re-inspections of their removal systems required, and a fine as a result of non-compliance with these Rules and Regulations.  The charge for re-inspections and fines for non-compliance will be established by the District Board of Directors.  Failure of the business owner/operator to pay the fines and costs for damages incurred will result in disconnection of water service to the premises from the District’s system and/or filing of a lien on the property.

Any business that has received two (2) consecutive violations shall be required to provide cleaning records to the District’s management office for a period of at least one year or longer until the business has had no violations for 12 consecutive months.  Failure to provide the required cleaning records may result in disconnection of water service to the premises from the District’s system and/or filing of a lien on the property.

*6

ARTICLE VIII

ANNUAL SEWER SERVICE FEES

- In order to cover the costs incurred by the District for operating, maintenance and capital replacement of the District’s sanitary sewer system, the District assesses sewer service fees to its customers pursuant to the Colorado Special District Act, C.R.S. Section '32-1-1001(1)(j).

*4

- The District renders invoices annually for its customers in February of each year.  Payment of sewer service fees is due thirty (30) days from the date of the invoice.  If the payment due date falls on a Saturday, Sunday or legal holiday, the payment due date will be the next regular business day.  Invoices for fees and any other notices are effective upon mailing said invoice or notice to the service address or a known current mailing address of the user as shown on the District's records.

- The District's sewer service fees are based on the rates established by the Board of Directors.  The Board may increase or decrease the District’s rates at any time.

- Service is presumed to be activated as of the date the sewer tap is inspected and approved by the District for the structure or premises.  In the event that a user can prove to the District that a delay of more than six (6) months has occurred from the date the sewer tap was inspected and approved by the District Engineer to the actual commencement of service, charges will begin on the date of actual commencement of service.  The date for the actual commencement of service is the date the certificate of occupancy was granted for each structure or premises.  A copy of the certificate of occupancy must be provided to the District Manager.

- All requests for transfer, termination or reactivation of District sewer service must be made in writing.  The District’s Utility Transfer Form (see Exhibit H) or Billing Information Change Request Form (see Exhibit I) must be used for all transfer requests and the District’s Utility Transfer Fee must be paid.  Not withstanding the requirements for written notification, the current owner of a property will be liable for all charges imposed by the District on the property to which services are provided from the date such charges become due until such charges are paid and such charges will be a lien on the property served.

*4

- Residential customers will be charged a flat rate annual sewer service fee.  Flat rates are determined using the District’s annual base rate and the District’s prior year average annual water usage volumes (AAWU) for the District’s residential developments.  The District’s annual base rate is based upon the expenditures projected for the District for the ensuing year.

*8	Commercial customers will be charged an annual sewer service fee based on the AAWU for their specific location multiplied by the District’s annual base rate.  When a commercial customer is new in the District such that actual water meter readings are not available to determine the customer’s AAWU, the AAWU will be estimated using actual water meter readings for other commercial customers of similar type.  Once actual water meter readings are available for the new commercial customer to determine their actual AAWU, an adjustment will be made to the annual sewer service fee for the following year.

*4

- Service charges for which payments are not received by the due date will be deemed past due and subject to interest and delinquent charges according to the District’s current fee schedule.  All customer accounts with past due charges will be sent periodic billing statements on which interest and delinquent charges will be assessed as applicable.  Whenever a billing statement includes interest and delinquent charges, the entire amount of the bill, including the interest and delinquent charges must be paid in full.

*4

- If a customer’s account has past due charges that remain unpaid ninety (90) days past the initial billing date, the District may initiate procedures for the discontinuation of service in accordance with Section 4, Article II of these Rules and Regulations and/or the District may file a lien on the property for which the account is delinquent in accordance with Section 4, Article III.

ARTICLE IX

DRY CLEANING FACILITIES

- All dry cleaning facilities shall be inspected by the District on an annual basis.  The operator shall maintain a summary of all cleaning chemicals used at the facility including the volume purchased and the volume recycled.  Spill control plans must also be maintained at each facility, including provisions for preventing any spill from entering the floor drains.

Violation of this section may subject the operator to fines by the District, Metro Wastewater Reclamation District and the USEPA.

ARTICLE X

MOLYBDENUM DISCHARGES

- All discharges from cooling towers, boilers, closed-loop heat transfer systems, and any other cooling/heating system treated with molybdenum-containing water treatment chemicals are prohibited.  The Metro Wastewater Reclamation District regulations issued pursuant to the Federal Clean Water Act and the Colorado Water Quality Control Act provide that any persons violating this prohibition within the Metro District’s service area may be subject to penalties of up to $32,500 per day for each such discharge.

SECTION 7

WATER SERVICE

###### ARTICLE 1

LICENSES FOR WATER SERVICE

- The right to take and use water within the District, distributed through the facilities of the Denver Water Department, exists and shall exist only under license and no physical connection may be made or modified to any such facilities or to any privately or publicly owned extension thereof for any purpose unless a license shall have first been obtained authorizing the use for which such a connection is to be made.

- Requirements for obtaining a license for a water service will be governed by the Rules and Regulations of the Denver Water Department and the District.

- TYPES OF LICENSES:  The District may approve the following types of licenses for water service as defined in the Denver Water Department's operating rules:

  - Metered Service License

  - Fire Protection Service License

  - Temporary Service License

  - License for Stub-In

- The issuance of licenses for water service within the district will be subject to any limits established pursuant to the District’s Total Service Contract with the Denver Board of Water Commissioners.

## ARTICLE II

FEES AND CHARGES

- No license for water service will be approved by the District without prior payment of the District's system development fee and the Denver Water Department's system development charge unless said license is issued for fire protection service or temporary service.

- A schedule of system development charges is established by the Board and is subject to periodic review and amendment as the Board deems appropriate.  The District’s schedule for water system development fees can be found on the District’s Fee Schedule.

- The District will defer a system development charge for a water service stub-in requested if no meter is to be set, through which no water will be used and which is only being installed to permit street paving.  Upon obtaining a stub-in agreement from the Denver Water Department, the applicant for stub-in water service shall submit the executed agreement to the District for approval.

Upon the owner requesting the conversion of a stub-in agreement to licensed water service, the owner shall pay the District's system development charge in effect at the time the conversion of the stub-in is requested.

Failure to obtain prior approval from the District for conversion of the stub-in and pay the District's then current system development charge prior to the expiration date of the stub-in agreement will result in the cancellation of the agreement and at the discretion of Denver and the District, disconnection of the stub-in.

- Payment of the District's system development charges and the District's approval of a water service license or stub-in agreement does not constitute an authorized license for water service until applicant has also paid all fees and charges assessed by the Denver Water Department and obtained its approval of the license for water service.

ARTICLE III

APPLICATION PROCEDURES FOR OBTAINING A WATER SERVICE LICENSE

- License applications for water service to a property within the Southwest Suburban Denver Water and Sanitation District is that which is outside the boundary of the City and County of Denver and are available from the Denver Water Department or the District Manager (See Exhibit F).

- Applications for water service will be filled out as follows:

  - LIGHT GREEN SHADED AREAS:  The owner-developer shall fill in all information requested in the light green shaded areas on the application as required by the rules of the Denver Water Department.  Attach a copy of the full legal description for the property/building if it will not fit in the area provided on the application.  The Southwest Suburban Denver Water and Sanitation District's Contract Number with the Denver Water Department is TS27 and should be placed in the blank provided.

  - AREA TO BE COMPLETED BY DISTRIBUTOR:  Once the applicant has paid the Southwest Suburban Denver Water and Sanitation District's system development fee for the size of tap indicated on the application, a District representative will sign and date the application authorizing the tap size indicated.  The application will be marked "PAID" along with the amount, the check number and the date of the payment.

- After the appropriate system development fees have been paid and the license application signed by a District representative, a copy of the top page of the application will be made by the District for its files and the original application returned to the applicant for submittal to the Denver Water Department for final approval.

### ARTICLE IV

SUSPENSION, REVOCATION AND CANCELLATION OF LICENSES

- Any license authorized by the District may be suspended or revoked by the Denver Water Department without obligation on it or the District to refund or repay any consideration which may have been given for the issuance of such license for reasons established by the rules and regulations of the Denver Water Department and/or the District.

- If water service to a licensed premise is not activated within two years from the date of the application for such water service, the license will be canceled.  In order to reactivate the license, applicant will be required to pay a reactivation fee and additional system development charges if an increase in such charges have been imposed by the District since the license was canceled.  If within one year from the date the application was approved by the District, a licensee requests the District cancel the water license, the District will refund the system development charge paid, without interest, and void the license upon receipt of written confirmation from the Denver Water Department that the tap has been disconnected from the main.  Whenever a license is voided, a new tap application is required prior to obtaining water service.

- If a stub-in is not converted to a tap within two (2) years from the date of the application, the stub-in agreement will be canceled by the District.

- Any license for water service approved by the District which, subsequent to activation, is not used (as defined in Denver Water's operating rules) for a period of five (5) consecutive years will be canceled by the District and no refund of system development charges will be made.

ARTICLE V

WATER SERVICE CHARGES

- Water service will be furnished only to persons residing in, or whose property is included within the District and subject to the Rules and Regulations of the District and the Denver Water Department.

- An applicant owning land outside the boundary of the District who desires service may petition the District to include the parcel and all of the applicant’s land contiguous to the parcel upon which service is desired within the District.

- Water service to all licensed premises is subject to the rates and billing procedures established by the Denver Water Department.  Charges for water service are normally billed by Denver Water for bimonthly periods of service.  A schedule of Denver Water's service rates can be obtained from Denver Water, which may increase or decrease rates without notice.

- It is a requirement of the Denver Water Department that whoever seeks water service must assume the obligation to keep such license in force by paying all charges against the licensed premises.  No water need be furnished to any licensed premises against which any charges remain unpaid.

ARTICLE VI

SERVICE LINES AND TAPS

- Connection to sources of domestic water supply or systems other than those approved by Denver and the District is not allowed.  This provision shall not be construed to prohibit the use and installation of separate and independent non-potable irrigation systems not connected to the District’s water or sanitary sewer systems.

- Notwithstanding the issuance of a license for water service, no connection may be made to any main carrying water from the Water System except as authorized by the District and Denver Water.  All taps through which water service is rendered by the District will be made by employees of the Water Department, except if contractually negotiated otherwise with Denver Water.

- No tap may be made to any main carrying water from the Water System until the service line with fittings and a meter installed.  The only exception to this is if a stub-in is necessary to install a service line prior to the paving of streets, in which case, a meter need not be set until water is actually used.

- All cost and expense incident to the installation and connection of the water service will be borne by the owner.  The owner shall indemnify the Denver Water Department and the District for any loss or damage that may directly or indirectly be occasioned by the installation of the water service.

- The maintenance and protection of privately owned service lines, fixtures and water-using appliances is the exclusive responsibility of the owner thereof.  The District and the Water Department are not responsible or liable for damage from any cause whatsoever to such service lines, fixtures and water-using appliances, and no licensee is entitled to reimbursement for damages or payment of refunds by reason of pressure changes or stoppage of the flow of water through the water system.

- A separate and independent water service line will be provided for every detached single-family residence.  No tap will be made to supply water service through a single service pipe for more than one licensed premises.

- Water service lines will be installed and constructed in accordance with Denver Water's Rules and Regulations.

- No unauthorized person shall maliciously, willfully, or negligently break, damage, destroy, uncover, deface or tamper with any structure, appurtenance or equipment which is a part of the water system.

- No connection between the water system and the water facilities of a licensee may be made except in a public street or similar location to which the Denver Water Department has as free a right of access as it would have in a public street.

- The Water Department will specify the water main to be tapped at the time of application for water service.

- Arrangements for tapping of the water main will be made with the Denver Water Department not less than 24 hours before the tap is to be made.

SECTION 8

EXHIBITS*5

A.		APPLICATION AND AGREEMENT FOR EXTENSION OF WATER MAINS

B.		APPLICATION AND AGREEMENT FOR EXTENSION OF SEWER MAINS

C.		SANITARY SEWER EASEMENT DEED

D.		APPLICATION FOR SEWER TAP PERMIT

E.		APPLICATION FOR SANITARY SEWER STUB-IN PERMIT and EXHIBIT A

F.		DENVER WATER DEPARTMENT APPLICATION FOR WATER SUPPLY LICENSE OUTSIDE THE CITY AND COUNTY OF DENVER

G.		LICENSE AGREEMENT FOR CROSSING EXISTING SEWER MAINS

H.		UTILITY TRANSFER FORM

I.		BILLING INFORMATION CHANGE REQUEST

EXHIBIT A

APPLICATION AND AGREEMENT FOR

EXTENSION OF WATER MAINS

EXHIBIT B

APPLICATION AND AGREEMENT FOR

EXTENSION OF SEWER MAINS

EXHIBIT C

SANITARY SEWER EASEMENT DEED

EXHIBIT D

APPLICATION FOR SEWER TAP PERMIT

EXHIBIT E

APPLICATION FOR SANITARY SEWER STUB-IN PERMIT

and EXHIBIT A

EXHIBIT F

DENVER WATER DEPARTMENT

APPLICATION FOR WATER SUPPLY LICENSE

OUTSIDE THE CITY AND COUNTY OF DENVER

EXHIBIT G

LICENSE AGREEMENT FOR CROSSING EXISTING SEWER MAINS

EXHIBIT H

UTILITY TRANSFER FORM

EXHIBIT I

BILLING INFORMATION CHANGE REQUEST FORM

SECTION 9

SPECIFICATIONS AND STANDARD DETAILS

E-STND	EASEMENT STANDARDS

G-NOTE	GENERAL NOTES

R-BLK	REVIEW BLOCK

S-1		TYPICAL SANITARY

S-2		BASE CHANNELIZATION

S-3		MANHOLE

S-4		MANHOLE RING AND COVER

S-5		MANHOLE STEPS

S-6		DROP MANHOLE

S-7		DOMESTIC SEWER TAP

S-8		KNOCK-OUT MANHOLE